.po 10

.he#Turbo DOS 1.42 8086 Update                T-Function 37#

.fo#Programmer's Guide         5-40-#         SET COMM BAUD RATE#

Turbo DOS 1.42 Update

Prepared by Advanced Digital Corporation

________________________________________________________________

#T-Function 37     Set Comm Baud Rate#

Entry Arguments     |-------------------------------------------|

| Reg |        Description                  |

|-------------------------------------------|

|  CL   = 37                                |

|  DH   = channel, plus bit 7 if remote     |

|  DL   = baud rate code (bits 3-0)         |

|         0 for   50 baud  8 for  1800 baud |

|         1 for   75 baud  9 for  2400 baud |

|         2 for  110 baud  A for  2400 baud |

|         3 for  134.5     B for  3600 baud |

|         4 for  150 baud  C for  4800 baud |

|         5 for  300 baud  D for  7200 baud |

|         6 for  600 baud  E for  9600 baud |

|         7 for 1200 baud  F for 19200 baud |

|                                           |

|         bit 7 = 1 for attention dection   |

|         bit 6 = 1 for 'CTS' handshake     |

|         bit 5 = 1 for input disabling     |

| (ADC)   bit 4 = 1 for Xon/Xoff Protocol   |

|___________________________________________|

Explanatio�        The Set Comm Baud rate function sets the baud

rate and options passed in the DL register for

the specified serial channel.

Bit 7 = 1 enables the serial I/O driver to

detect the Turbo DOS defined keyboard attention

sequence (#ATNCHR# patch point).

Bit 6 = 1 enable the ADC 'CTS' handshake

between the serial I/O driver and the

serial device.  PS-NET/I the DB-25 pin

20 is routed to the DCD input of the serial

controller����  Th堠� seria젠 I/Ϡ�� drive� �

interrprets the negation of the DB-25 pin 20

signal (OFF condition <-3VDC) as not clear to

send��  Th� PS-NET/Mode� provide� th� DB-2� �

pin 5 (CTS) signal to the CTS input of the

serial I/O controller.  The serial I/O driver

interrprets the negation of the DB-25 pin 5

signal (OFF condition <-3VDC) as not clear

to send.  The PS-NET/RS-422 provides the DB-9

pin 4 to the CTS input of the serial I/O

controller����  Th堠� seria젠 I/Ϡ�� drive� �

interrprets the negation of the DB-9 pin 4

as not clear to send.�

Bit 5 = 1 disables the input section of the

serial I/O driver.  All characters received

by the serial I/O drivers are accepted and

ignored.

Bit 4 = 1 enables the Xon/Xoff protocol for

serial printer drivers.

�

Note: that the option bits are a function of the

eletrica� interfac� interconnectio� betwee� � �

seria� devic� an� � seria� devic� controller.

#*************

*  CAUTION  *

*************#

User� o� thi� functio� ar� requeste� t� �

revie� th� exac� interconnectio� fro� � �

seria� devic� t� � leve� converte� (PӠ NET� �

to a serial I/O controller. Further, the user

should inspect the source for the serial I/O

driver to determine the precise effect the

settin� o� a� optio� bi� wil� hav� o� � �

serial device.

#####