Z-80 Floppy Disk Test

Copyright (c) 1980 by Laboratory Microsystems

4147 Beethoven Street

Los Angeles, California 90066

1.0  General description

The  Z-80 Floppy Disk Test is a general purpose utility for testing or

initializing a diskette.  It requires a Z-80 microprocessor, at  least

16  kbytes  of random access memory, two flexible disk drives, and the

Digital Research CP/M 2.x operating system.  Program mode and area  of

diskette to be tested are controlled from the keyboard.  Errors may be

audited on the console or the line printer.

As supplied on the distribution diskette, the program is configured to

test  a  soft-sectored, single density disk with 77 tracks, 26 sectors

per track, and 128 byte sectors.  If you are using a different type of

disk drive, the number of tracks  and/or  sectors  per  track  may  be

specified during the initialization dialogue.

The program does not exhaustively check out the disk  drive,  and  any

errors  detected  by  the  program  must  be  interpreted  with  care.

Intermittent  errors  occurring  throughout  a  broad  range  of  disk

addresses  probably  indicate  a  problem  with  the  disk drive unit.

Errors indicated at consistent addresses within  adjacent  tracks  may

result  from a worn or damaged flexible disk.  Error free execution of

the test certainly indicates an intact and usable diskette,  and  also

is  fairly good evidence of proper operation of your disk drive.  Note

that the program is rather insensitive to drive  problems  related  to

head  movement, since the diskette is addressed in a smooth sequential

manner.

2.0  Installation

Mount  your system disk in drive A, and the distribution disk in drive

B.  Use the PIP utility to transfer the test program onto  the  system

disk:

A>pip a:=b:dtst.com [v]

Now  remove  the  distribution  disk  from drive B and store it as the

backup copy.

3.0  Using the Test Program

Put your system disk in drive A, boot up the CP/M operating system, and

type:

A>dtst

The test program will load  into  memory  and  print  the  identifying

message:

Laboratory Microsystems Disk Test version 1.0

If there is inadequate memory in the transient program area, or if the

operating  system is not CP/M version 2.0 or greater, the program will

immediately print a warning message and  exit.   Otherwise,  the  test

proceeds  to  the initialization dialogue.  The test mode is set via a

series of questions presented on the system console.  Answers  calling

for  an  alphabetic  character  may be upper or lower case; a carriage

return is not required.  Answers calling for  a  number  must  be  two

digits, or one digit followed by a carriage return.

query                   responses       explanation

-----                   ---------       -----------

Itemize errors?         Y or N          If   answered  y,  each  error

detected  is  audited  on  the

selected  device.  If answered

N, only the total error  count

is  printed at the end of each

pass.

Console or Printer?     C or P          If  itemizing  errors,  select

the   device   on   which  the

itemized error listing will be

printed.

Lock on read or write?  N, R, or W      N = no locks.   R  =  lock  on

read  mode (no data is written

to the  disk  ---  useful  for

detecting any damaged areas on

the media).  W = lock on write

(all   data  on  the  disk  is

destroyed --- this may be used

to erase a  diskette  by  also

locking on pattern)

Restore original data?  Y or N          Y  =  restore existing data on

diskette  after  testing  each

sector.   N  =  do not restore

existing data.  Test runs much

faster when  restore  mode  is

not used.

Lock on data pattern?   Y or N          Y  =  user  wishes  to specify

specific data to  be  used  as

the  test pattern.  N = random

data pattern is  generated  by

the test program.

Enter data pattern      hex 00-FF       If  you  answered Y to Lock On

Pattern, enter the  eight  bit

data pattern to be used during

disk testing.  If you answered

N, this question is bypassed.

Drive to be tested      B, C, or D      Specify  disk drive to be used

for  diskette   testing.    In

order  to  prevent  accidental

destruction  of   the   system

disk,  the  use  of drive A is

not allowed.

Test all tracks?        Y or N          Y = test all tracks (00-76) in

sequence.  N = user wishes  to

specify  range  of tracks (may

be used to test only part of a

diskette, or if disk drive  is

other  than  standard  8  inch

single density).

First track to test     decimal 00-76   If you answered N to Test  All

Tracks,   specify   here   the

number of the first  track  to

test.

Last track to test      decimal 00-76   If  you answered N to Test All

Tracks,   specify   here   the

number  of  the  last track to

test.  Must  be  the  same  or

greater than the number of the

first track to test.

Test all sectors?       Y or N          Y = test all sectors (1-26) in

sequence.   N = user wishes to

specify range of sectors  (may

be  used  to test only part of

each track, or if  disk  drive

is  other than standard 8 inch

single density).

First sector to test    decimal 01-26   If you answered N to Test  All

Sectors,   specify   here  the

number of the first sector  to

test on each track.

Last sector to test     decimal 01-26   If  you answered N to Test All

Sectors,  specify   here   the

number  of  the last sector to

test on each track.   Must  be

the  same  or greater than the

number of the first sector  to

test.

How many test passes?   decimal 01-99   Enter   the   number  of  test

passes  to  be  made  on   the

diskette.   At  least one pass

is always  made  even  if  you

enter zero.

After entry of the number of  test  passes,  the  program  immediately

begins  executing.  The program may be interrupted by pushing any key,

however response may be delayed for several seconds depending  on  the

speed  of  the  processor  and  disk  drive.   Do not stop the test by

resetting the computer system  if  you  are  using  restore  mode,  or

unpredictable amounts of data may be lost.

4.0  Error Listing

Errors  detected  by the program may be itemized on the system console

or the line printer.  The pass, drive name, track,  and  sector  where

the error occurred are listed.  All numbers are in decimal.  The error

type is designated at the right.  In the explanations below, "hardware

error" signifies an error status code returned from the low level disk

driver, and usually means a CRC fault or seek incomplete.

error type                      explanation

----------                      -----------

read error - original data      If  running  in restore mode, hardware

read error noted as the original  data

was being loaded into memory.

write error - test data         Hardware  error noted as the test data

pattern  was  being  written  on   the

diskette.

read error - test data          Hardware  error noted as the test data

pattern was being read back  from  the

diskette.

compare error - test data       Difference  was found between the data

pattern  that  was  written   to   the

diskette  and  the  data that was read

back.

write error - restore phase     Hardware error noted as  the  original

data  was  being  written  back to the

diskette.

read error - restore phase      Hardware error noted as  the  restored

original  data was being read back for

verification.

data cannot be restored         Compare  error   found   between   the

original   data   rewritten   to   the

diskette and  the  same  sectors  read

back for verification.

5.0  Program areas of interest to users

To  use  the  program  on  other  types  of disk drives, it may become

necessary to change the first two locations given below.  This may  be

done  be  reading  in  the  COM  file  with  DDT, patching the desired

location, and SAVEing a new COM file.

0119H   (8  bits)  Sector skew flag.  If byte=0, skew is not used.  If

byte=1,  remap  sector  numbers  as  given   in   the   sector

translation  table.   On  Z-80  systems  at 4 mhz, skew is not

needed and this byte may be left zero.  On systems with slower

processors, test speed may be improved by use of skew.

0121H   (8  bits)  Contains the number of digits to be accepted by the

numeric input routines during the initialization dialogue, the

value is two in the distributed version.  If test is to run on

a drive with greater than 99 tracks, change the value of  this

byte to three.

0122H   (16  bits)  Contains  the  address  of  the sector translation

table.  In the distributed program, the table is built with  a

skew factor of three, and is 26 bytes long.

#########################################################################################################################