** ERROR STATEMENT DOCUMENTATION

3 TYPE ERRORS EXSISTS.

1.FATAL ERROR

1-1	NOT ENOUGH MEMORY FOR EXECUTION

1-2	LABEL TABLE OVF.

1-3	NO SOURCE FILE

1-4	DISK OVER FLOW

1-5	ORG,DEFS ERROR (THE VALUE IS NEGATIVE

OR IT REFERED UNDEFINED LABEL.)

THE FATAL ERROR STOP ALL EXECUTION.

AND CLOSE ALL FILES,THEN REBOOT CP/M.

2.EXECUTION ERROR

NORMALY THIS ERROR DOES NOT OCCUR

BUT THE DEFECTIVE AX68K.COM PROGRAM.

IF USER REVISION IS NOT CORRECT,THEN THIS ERROR MAY OCCUR.?

3.SOURCE PROGRAM ERROR

ODD BOUNDARY,DOUBLE DEF.LABEL,SYNTAX,ETC. ARE OUTPUT TO .PRN FILE OR CONSOLE.

AS FAR AS THE NON FATAL ERROR FOR TRANSLATION

THE OP-CODE WILL BE MADE EVEN IF THE ERROR STATEMENTS ARE OUTPUT.

ORDINARILY, THE UNDEFINED LABELS ARE INTERPRETED AS TO 00000000H.

WHEN THE ODD BOUNDARY ERROR,DIFFERENT FROM OTHER ERROR,OCCURS

,THE ERROR STATEMENT LEADS THE OPCODE.

(OTHER ERROR STATEMENTS ARE THE POST-STATEMENT OF THE OP-CODE LIST.)

IF THE ODD BOUNDARY ERROR OCCUR,

THEN PC IS INCREMENTED 1 BYTE LONG.

AND THE 1 BYTE 00H IS INSERTED AUTOMATICALLY.

Each instruction of MC68000 has individual data size

and restrictive operands because of the enhancement of the software reliability.

Therefore the unsuitable usage of the size or addressing

will cause error.

The error traps the CPU.

However in this assembler, the syntax errors are swept away

by the error statement as much as possible in the asseble phase

, before the actual execution with the CPU.

ex. (Consistency of default size indicator.)

ABCD	D1,D2	;NO ERROR (size=BYTE)

ABCD.L	D1,D2	;SIZE ERROR

ABCD.B	D1,D2	;NO ERROR

EXG	A3,D5	;size=LONG

LEA	(A6),A5	;size=LONG

BRA	$(LABEL) ;size=BYTE

BRA.W	$(LABEL) ;size=WORD

etc.

(attention.	Some operation do not allow some data sizes.

therefore careless usage of size indicator will cause error message.)

In many cases, even if you think that you are correct,

this assmbler may output an error message. ?

In those cases, please read MC68000 instruction manual precisely.

3-1. EXPRESSION SIZE of the operand

As formerly descrived, the expression value of numerals

are 32-bit long internally.

So, the following expression will cause OVERFLOW ERROR.

The Occurrence of the error depends on the size to be extracted.

For example, suppose that

AAA	EQU	0FFFFFFH

This means that internal data of AAA=00FFFFFFH=+16777215 ,

because the value is defined as 32-bit numerals initially.

And this does not mean AAA=0FFFFFFFFH=-1 .

So, the extraction by the use of the word size indicator

may cause the overflow error.

But in the next example, although the error statement were output,

the assembled macine-code will be correctly work.

Because the 16-bit effective address will be sign extended by the CPU.

And output address data is 24-bit long.

ex.

PANEL	EQU	0FFF000H	;PANEL SWITCH INPUT PORT

LEDDISP	EQU	0FFF001H	;LED DISPLAY OUTPUT PORT

MOVE	PANEL,D0

MOVE	D0,LEDDISP

;(NO SIZE INDICATOR IS INTERPLETED AS WORD SIZE.)

If the user does not like this error statement outputs,

please use CLOSEOVF pseudo-operation,

which will suppress overflow error statements.

But this procedure will hide true overflow error too.

(If you wish to re-open the error statement output, use OPENOVF pseudo-op.)

** Additional informations are contained in the TEST source files in this diskette.

And will be with your original sources.

Prease examine those.

##############################################################################################################